Version 3.1	February 2003.

Mr.Atari's MyIDE(tm)-hardware for ATARI XL/XE-computers.

THIS MANUAL CONTAINS STEP BY STEP INSTRUCTIONS HOW TO USE THE SUPPLIED SOFTWARE. READ, ENJOY AND HAVE FUN!

Content:

- Checking your harddrive, SCANDISK.BAS and UNICHK.BAS

- Partitioning your harddrive, FDISK.BAS

- How to work with your system and harddrive.

- How to make and work with IMAGES.

- Extra keystrokes.

- Creating your own MASTERdisk.

- Create your own bootable disk and load the OS from harddrive.

! Before you start, please make a backup of the original disks!

1.	Checking your harddrive, SCANDISK.BAS and CHKDSK.BAS

Before you can start using a harddrive, you have to check if it is compatible with the MyIDE-interface. I know this is a strange topic, but somehow not all IDE-drives follow the ATA-specifications I have used to design the interface and software. But don't worry too much about this, 90% of the drives will work.

Boot the UTILITYdisk with BASIC enabled. Then type RUN"D:SCANDISK.BAS". The program detects the interface (internal/external) and does a detection of sectors, heads and cylinders. At this point, most of the incompatible or bad harddrives will fail to give the correct parameters. Then you can start a deep scan (check all sectors), heads only (check only sector’s 1), sectors only (check only head’s 0) or quick scan (check only head’s 0, sector’s 1). Quick scan will do fine, but if you want to be 100% sure, you need a deep scan. Since an Atari in only 1.7 Mhz a deep scan goes as fast as 20 Mb/hour. Quick scan normally runs up to 20Gb/hour.

If you want to write, read and verify your harddrive, you need to run UNICHK.BAS. This program needs manual input. First question is type of interface, input the correct (I or E) type. Then the number of sectors, heads and cylinders are asked. You can find them on the label of your harddrive. After this you are asked if you are sure that you want to write to the drive (this will destroy all data!) or only read / verify it. On the screen an error log will appear if there are any. With writing enabled, you completely erase the drive. Filling all sectors with a certain bit pattern.

When you have checked your harddrive, you can proceed with the next chapter.

2.	Partitioning your harddrive, FDISK.BAS

Partitioning your harddrive on your Atari is basically the same as on your PC. At this point you need to decide how many logical drives you want to have on it, what device numbers and were the starting point is for disk images.

Remember that a partition called D1: will disable the floppy-drive connected as D1:, any partition Dx: will disable the floppy at Dx:. Pressing SHIFT+CONTROL+D will disable the IDE-patch en consequently free up all floppy-drives.

Making Partitions:

Boot the UTILITYdisk with BASIC enabled. Then type RUN"D:FDISK.BAS". FDISK will auto-detect the interface and if the harddrive has MyIDE data on it. If the harddrive is new, the software will ask you to initialize it. Press Y enter and wait.

Menu:

1 Drive parameters. 1 (enter) followed by Autodetect, Manual input or Quit.

Normally this option is not used, since the software will auto-detect the first time you run FDISK. But in case you want to do the detection or manual input, this menu item is present.

2 Erase partitions. 2 (enter) followed by Y

Used to erase the partition tables, option-byte and image-loading (sector 1). Data present on the harddrive is not erased and can be accessed again by putting partitions back (with exactly the same sizes). Drive numbers may be different.

3 Partitons <-> Parameters. 3 (enter)

The information on the screen is changed from parameter-display to partition-display.

On the parameter display you can see how many cylinders, head and sectors your harddrive has. How many partitions are present, were images are stored and how many cylinders 1 image consumes. When you want to see more details about the partitions, you need to change to the partition-display. On the partition-display you can see: Drive number, starting cylinder, ending cylinder, size in sectors (*), size in Kbytes and density of the sectors. (*) This is the amount of sectors physical present on the claimed cylinders. You can ignore the higher value or configure your MyDOS to use these otherwise wasted sectors.

4 Make partition. 4 (enter) followed by manual input.

The first partition you create always starts at cylinder 1, all further partitions are put in sequence behind each other. Except when the image-loading is set, then the space above is used as default, unless you specify to occupy space below the images. If you want to make a 65535 double density sectored drive called D3: (16 Mbytes), you enter: 3 (enter) 65535 (enter) 1 (enter). If this partition is bigger then the room available a message "disk full" appear shortly. The size is then modified to fit the room left.

5 Delete partition. 5 (enter) followed by Y.

The last partition created will be deleted.

6 Image-loading (setup/off). 6 (enter) [(enter)]

When the image-loading is being set and there is room for it, it will ask for the starting cylinder highbyte. As default the lowest possible cylinder will be filled in. Selecting 6 (enter) a second time will disable image-loading.

7 Optionbyte.

Here you can set your preferences for booting / handling MyIDE

8 Save & exit. 7 (enter).

Quit and save changes.

9 Exit. 8 (enter).

Quit without saving.

3.	How to work with your system and harddrive.

After you have partitioned your harddrive you can work with it. Please remember to configure your (newly made) MyDOS-disk were you are planning to boot from! (this can be a partition on your harddrive). Enter the correct amount of sectors for your partitions (high capacity drive) and write the DOS/DUP files to your disk. Option O in the DOS menu, enter: drive-#, N, Y, Y, sectors#, followed by H (write DOS files).

Since the Atari-system itself doesn't support IDE-drives it is best to turn on your Atari first let it go to SELFTEST or BASIC, then start your harddisk and other devices.

(When you start your harddisk first and then your Atari, the harddisk will sometimes lock-up, due to garbage send from the cold start routine to clear up and initialize the custom chips).

Then insert the proper side of the MASTERdisk and boot it, with or without BASIC.

Side A: for INTERNAL interfaces.

Side B: for EXTERNAL/CARTRIDGE interfaces.

(Or the self made disk to load the OS from HD).

You can enter the image-routine by holding START until the boot process is finished.

(Remember to hold OPTION to disable BASIC until the booting starts, then release OPTION and hold START).

After booting the MASTERdisk/HD the MyIDE-patch text will appear and you will be prompted to insert your boot disk and press SELECT (unless disabled). Put in the disk you want to work with and press SELECT. If you have a partition D1:, OR, redirected to boot from a other diskdrive from your harddisk, you can push SELECT straight away.

After a complete boot, you can enter the image-routine at anytime by holding START and pressing RESET. Remember that you can not disable/enable BASIC anymore at this point with the OPTION key.

Tips:

- When your harddrive locks-up (for any reason), simply press reset. This will normally get it free, and if not, your system will deselect the harddisk preventing your OS from crashing. When RESET doesn't work: power down/up your harddisk. Preferably with the IDE-cable disconnected. Then hook it back on and press RESET again.

- When you made a (new/empty) partition D1: on your harddrive, you cannot boot the MASTERdisk and continue the boot. Simply because your partition D1: isn't bootable yet. Boot the MASTERdisk with basic disabled. Press shift+control+D (to disable the MyIDE-patch) when the ‘press SELECT’ message appears. Then continue the boot-process with your UTILITYdisk, the DOS-menu will appear. Activate the MyIDE-patch by pressing shift+control+E. Now you can format and write the (configured) DOS-files to your D1: harddisk partition.

- Remember to configure MyDOS with the correct size high capacity drives and then write these DOS files to your boot disk.

- Remember to set the Optionbyte ONLY when you have a running system and are sure that the selected options are available.

- A faster way of booting is when you write a copy of the MyIDE-patch to your harddisk and create a bootdisk for this. Even faster is when you also redirect the boot to a harddisk partition AND disabled the ‘press SELECT’ message. The UTILITYdisk has the needed software for this.

4	How to make and work with IMAGES.

Since DOS is limited (maximum 8 drives and 16Mb [MyDOS] or 180k [DDDOS]), you are never able to use the full capacity of your harddrive. Here the IMAGE-routine comes into perspective. A Image is a diskcopy of any disk you like, put into a slot (number) on the harddisk.

To activate IMAGE-loading, set this feature with FDISK. FDISK will check if there is enough room and select a safe space for you to use. FDISK handles images as a large partition and keeps it safe when you make changes in partitions when you use FDISK again and again.

(You need to reboot your system after using FDISK).

Once set, you can enter IMAGE-load by holding START and pressing RESET.

>How do I copy disks to images?

Enter the IMAGE-routine and select a slot. Put the disk into drive #1 (write protect it!) and move the joystick LEFT while pressing the FIRE button. After the disk has been copied, the counter will be reset to 0001.

>How do I copy images to disks?

Enter the IMAGE-routine and select a slot. Put a empty formatted disk into drive #1 and move the joystick RIGHT while pressing the FIRE button. After the disk has been copied, the counter will be reset to 0001.

>How do I load a image?

Best is to cold start your Atari.

Start the booting of your MASTERdisk (or from HD) and hold OPTION if BASIC is unwanted. Once the boot starts, release OPTION, press and hold START, wait until the boot finishes.

The image-counter will appear.

Select a slot (move your joystick UP/DOWN)

Press the FIRE button.

>How do I flip a disk in a multi-diskette game?

When the program asks you to change disks, press SHIFT-CONTROL-number.

Where number equals 1 for side 1, 2 for side 2, etc. (maximum is 9)

(Remember that you have to copy the disks to images in sequence!)

Note:

- Not all images (mostly games) will work correctly. This is due to the fact, that these games de-activate the MyIDE ‘soft’-OS and try to load from floppy instead. When you have a MyIDE ‘hard’-OS almost everything runs!

- You can only handle SD or MD disks. DD (256 bytes/sector) is not supported.

5	Extra keystrokes.

With extra keystrokes you can change the configuration of your MyIDE.

You can even set some as default with the option-byte in FDISK.

On screen Harddisk-activity.

shift+control+A toggles it on/off

Protect your harddisk from writing.

shift+control+P Protects your drive

shift+control+U Unprotect it.

Free your floppy drive.

shift+control+D Disables the MyIDE patch

shift+control+E Enables the MyIDE patch

Load disk-images from HD.

Hold START and press RESET to enter the image-routine

The image routine ALWAYS disables onscreen activity and protects your harddrive. You can change this with the described keystrokes.

Also you can swap disks in this mode by pressing shift+control+[number]

6	Creating your own MASTERdisk.

Before you start this, you MUST have the ‘press SELECT’ message enabled!

On the UTILITYdisk you find software to make a MASTERdisk. There are two stages in this process.

- Write a copy of the MyIDE-patch to the new MASTERdisk

- Make the new MASTERdisk bootable

First you have to boot the correct side MASTERdisk with basic disabled.

Press SHIFT+CONTOL+D to disable booting from HD.

Put the UTILITYdisk in the drive and press SELECT. The DOS menu will appear.

- Load the program WRITFL.OBJ, insert your new disk, press START

- Re-insert your UTILITYdisk.

- Press RESET

- Load the program BOOTFL.OBJ, insert your new disk, press START

- Label this new disk “MASTERdisk”

FINISHED!

Note:

In fact you can safe any operating system (you have made) this way. The only thing you have to remember is that location $C745 is the init routine and needs to contain a RTS minimum. The program WRITFL.OBJ will check if your OS is usable before writing. The boot-block you write with BOOTFL.OBJ loads the saved OS from sectors 4-131 and put this under the Atari-OS, activates the loaded OS and sets the reset-vectors.

You can create a copy of the Atari-OS to work on using ROMRAM.OBJ. After loading HPATCH.OBJ you have the ‘soft’ MyIDE-OS for a internal interface (HPEXT.OBJ for a external interface). The source ROMRAM.LST is on the disk too!

7.	Create your own bootable disk and load the MyIDE-patch from harddrive.

Before you start this, you MUST have the ‘press SELECT’ message enabled!

On the UTILITYdisk you find software to do this. There are two stages in this process.

- Write a copy of the MyIDE-patch to your harddisk

- Make a bootdisk

First you have to boot the correct side MASTERdisk with basic disabled.

Press SHIFT+CONTOL+D to disable booting from HD.

Put the UTILITYdisk in the drive and press SELECT. The DOS menu will appear.

- Load the program WRITHD.OBJ, press START

- Press RESET

- If you have a internal interface:
Load the program BOOTHDI.OBJ, insert your new disk, press START

- If you have a external interface:
Load the program BOOTHDE.OBJ, insert your new disk, press START

- Label this new disk “BOOTHD”

FINISHED!

This new bootdisk boots the MyIDE-OS from HD and is much faster!